3GPP TSG SA WG3LI	Tdoc S3LI01_140

October 30 - November 1, 2001

Aspen, Colorado, USA

# Source:		Alcatel S.A.

# Title:			LIC (Lawful Interception Correlation) Header

# Document for:	Discussion and Decision

Subject: 		Handover specifications for TS 33.108

Contact: 		laurent.perrine@alcatel.fr

# Introduction:

Nokia Corporation presented two ways to enhance GLIC header defined in 33.108:

- to define a dynamic header

- to define a static header with additional information

We, Alcatel, are supporting Nokia for defining a dynamic header, which seems the only way to define something future-proof.

Nevertheless, we are considering Nokia proposal is not ambitious enough.

This contribution is an enhancement of Nokia proposal in following ways:

- because there is no compatibility between GLIC V0 and GLIC V1, it is suggested to define a generic header with no more link to GPRS (and by the way with GTP protocol, avoiding spare octets or unexplainable parameters) called LIC (Lawful Interception Correlation).

- to avoid inconsistency between HI2 and HI3 (as there were for correlation number), it is suggested to define LIC header in ASN.1. Some will argue about terrific complexity of ASN.1: to avoid this kind of discussion, corresponding TLV according to BER is also presented.

LIC header ASN.1 definition:

lIC-header [1] LIC-header

LIC-header ::= SEQUENCE

{

lIC-version		[1]	LIC-Version,

lIID			[2] 	OCTET STRING (SIZE(1..25)),

correlation-Number	[3]	OCTET STRING (SIZE(8..20)),

t-PDU-direction	[4] 	TPDU-direction OPTIONAL,

sequence-number	[5]	INTEGER,

timeStamp		[6]	OCTET STRING (SIZE(12)) OPTIONAL,

-- YYMMDDhhmmss BCD coded

…

}

LIC-Version ::= ENUMERATED

{

version1(1),

…

}

TPDU-direction ::= ENUMERATED

{

from-target (1),

to-target (2),

unknown (3)

}

Corresponding BER encoding

If we've got:

- LIC header content

- LIID 			 "liid"

- correlation-number 	1122334455667788'H

- t-PDU-direction	from-target

- sequence-number	67'H

- time-stamp		'011024161500'

LIC-header TLV BER encoding is:

[A1]	[21]	[								lic-header

[81]	[01]	[01]						lic-version

[82]	[04]	[6C696964]					LIID

[83]	[08]	[1122334455667788]				correlation-number

[84]	[01]	[01]						tPDU-direction

[85]	[01]	[67]						sequence number

[86]	[06]	[011024161500]				time stamp

]

Another example with some fields missing:

- LIC header content

- correlation-number 	1122334455667788'H

- t-PDU-direction		to-target

LIC-header TLV BER encoding is:

[A1]	[10]	[								lic-header

[81]	[01]	[01]						lic-version

[83]	[08]	[1122334455667788]				correlation-number

[84]	[01]	[02]						tPDU-direction

]

It must noticed (for those afraid by CPU time consuming due to encoding/decoding) that tags and length (for parameter with a fixed size) can be hard-coded.

Conclusion

If 3GPP SA3 LI wants to define a robust, future-proof, consistent, non ambiguous, scalable, universal, …  data structure for HI3, referenced in other standards, Alcatel feeling is ASN.1 must be considered to define this data structure.