3GPP TSG SA WG3-LI						S3LI02_072

9-11 April 2002

Orlando, Florida, USA

Source:		Manfred  Beierer, Reg TP

Contact:		manfred.beierer@regtp.de

##### Title:			ASN.1 – Object Identifier

Document for:	Discussion/Approval

# Introduction

At the at the ASN.1 workshop at ETSI, 19.03.2002, the problem of how to use the object identier (OID) especially in context with FTP was identified. Based on the decision that every technology group should produce its own ASN.1 module, at the receiving (LEA) side some identifier is absolutely required to distinguish between the different modules. Different solution would be possible, e.g. to use a specific file name convention. But the most obvious solution in our opionion would be to use the OID. However, whereas the OSI protocol stack supports the use of OID, the internet protocol stack does not.

This contribution is intended to propose a possible solution.

# Use of OID in OSI

In the OSI protocol stack, the OIDs are used to identify the application. The OIDs are therefore transmitted not in the application itself but in the different layers of the OSI stack, especially in the presentation layer and the ACSE.

Enclosed (embedded in a zip.file) is an example, which is a trace from a FTAM session (Initialize Request). FTAM has the OID .. 8571 ... To clearly distinghish the layers, the presentation layer has been coloured yellow, the ACSE green and the FTAM blue. Please activate search function and search '8571' (OID) repeatedly.

# Use of OID in FTP

The priciple described in the previous section to address the application already in the underlying layers cannot be used in FTP, because the Internet Protocol Stack does not support the use of OIDs. There are only port numbers, e.g. 20 and 21 for FTP. Therefore it is proposed to transmitt the OID in the ASN.1 encoded file with the IRI data. To demonstrate from the ASN.1 source code up to the encoded file how this could be realized, three files are attached (see embedded zip.file):

| test.asn | This file contains the ASN.1 source code. The following two lines from the module of ES 201 671 are added and modified for the use in the 3GPP module: 1) -- Object Identifiers Definitions ------------------------------------------------------------------------------------ -- Security DomainId lawfulInterceptDomainId OBJECT IDENTIFIER ::= { ccitt (0) identified-organization (4) etsi (0) securityDomain (2) lawfulIntercept (2)} -- Security Subdomains threegppSUBDomainId OBJECT IDENTIFIER ::= {lawfulInterceptDomainId threeGPP (4)} hi2DomainId OBJECT IDENTIFIER ::= {threegppSUBDomainId hi2 (1)} and 2) hi2DomainId [24] OBJECT IDENTIFIER, Line 2) is inserted into the SEQUENCE of the IRI, thus hi2DomainId is part of the file with the IRI data. This parameter could be moved further down (within the curly brackets), but it would be better to have it at the first place in the file together with the version number (TAG 23), because when decoding the file this parameters is the first to be known. |
| --- | --- |
| test.doc | This file shows the encoded file in an human readable form (derived from an example of OSS Nokalva and using their compiler). To increase the understanding, implicit tagging has not been used for this example. After the code for sequence (30 h) and the length of the whole file (40 h) there is the context specific tag 24 (B8 h) with the length 8. The next byte contains the universal TAG of the Object Identifer (06) with the length 6. The next 6 bytes contain the value of the OID, encoded according to the BER (first two digits combined in one octet). The value used is the one assigned to the module of 3GPP by SEC LI, assuming that 1 will be used for HI2. Perhaps an additional leaf for the version would be useful. The same example using implicit tagging is also included in the Word file. |
| test.hex | This is the hex file which would be transmitted by the FTP and received by the LEMF. The LEMF when decoding the file first looks for the OID to have the knowledge which module was used for encoding and then continues decoding the individual parameters based on the appropriate moduole. |

# Proposal

The example demonstrates clearly that it is possible to transmit the object identifier within the IRI record. As the LEMF depends on an unambiguous identifer for the different modules, it is proposed to agree on the described insertion of the OID into the Module of 33.108. The corresponding change request could be drafted after agreement.

This contribution will also be sent to SEC LI  in order to propose this solution for the module of ES 201 671, too,  and to recommend it to the other technology groups, e.g. AT-D, TIPHON etc.