3GPP TSG SA WG3LI	Tdoc S3Li01_89

21-23 August, 2001

Saarbrücken, Germany

# Source:	Siemens AG

# Title:		LI Call Flows for IMS

# Document for:	Discussion/Information

Subject: 		Examples for LI Call Flows for IMS and LDI

# Introduction:

The intention of this contribution is to provide an example of available information (IRI/SIP-Messages) for interception in IMS. Also LDI (Location Dependent Interception) could be discussed with this example.

# LI Call Flow for :Session Redirection initiated by S-CSCF to general endpoint

The following Fig. is the adapted Figure 10.4.4-1: Session redirection initiated by S-CSCF to general endpoint out of TS 24.228 110. All details about available parameters in the SIP –Messages are also described at TS 24.228 110 at chapter 10.4.4.

According our requirements at TS 33.107 all SIP-Messages at P-CSCF and S-CSCF are forwarded to the relevant DF2. No location information is available at any of this messages; but this would be the main requirement to support LDI. In this case only the location of the CSCF could be the available location info; i.e. interception according location is supported via administering LI (the relevant SIP-URL) in the appropriate CSCF’s.

Within this example P-CSCF#1 and S-CSCF#1 are located in one country and S-CSCF#2 is located in an other country.