PROJECT:	T1P1 Activity on Lawful Intercept for UMTS

TITLE:	Recommended changes to TS 33.108 to address Issue #2, Packet Activity, of the issues list documented in T1P1/2002-004.

SOURCE:	Lou Degni

CALEA Implementation Section

14800 Conference Center Drive, Suite 300

Chantilly, VA 20151-3810

Tel:  	(703) 814-4729

Fax:  	(703) 814-4720

Sandra Lopez

Telcordia Technologies, Inc.

331 Newman Springs Road

Red Bank, NJ 07701

Tel:	(732) 758-3222

Fax:  	(732) 758-4196

e-mail:	slopez@telcordia.com

DATE:	January 22, 2002

LOCATION: 	Kiawah Island, South Carolina

ABSTRACT:	Proposed text to address Issue #3, Packet Activity of the TS 33.108 issues list documented in T1P1/2002-004.

DISTRIBUTION:	T1P1, T1P1.SAH

NOTICE

©2002 Telcordia Technologies, Inc. The proposals in this submission have been formulated to assist committee T1P1.  This document is offered to the committee as a basis for discussion and is not binding on Telcordia Technologies. The requirements are subject to change in form and numerical value after more study. Telcordia Technologies specifically reserves the right to add to, or amend, the quantitative statements made herein.  Nothing contained herein shall be construed as conferring by implication, estoppel, or otherwise any license or right under any patent, whether or not the use of information herein necessarily employs an invention of any existing or later issued patent.

[PAGE INTENTIONALLY LEFT BLANK]

A. Introduction

This contribution proposes text to address shortcomings in TS 33.108 in area of reporting Packet Activity events. This contribution addresses Issue # 2 of the issues list documented in T1P1/2002-004 and action items assigned to CIS/Telcordia during the T1P1.SAH conference calls held on January 8, 2002 and January 14, 2002.

This contribution builds on contribution T1P1/2001-132 R0 which was submitted for discussion to T1P1.SAH at it’s September 24-25, 2001 meeting in Dallas, Texas and on T1P1/2001-132 R1 which reflected actions taken by the authors to revise and resubmit as per recommendation from T1P1.SAH. The authors advise the readers to please see contribution T1P1/2001-132 R1 for additional reference material on this issue.

B. Discussion

Background: Similar to wireline systems, wireless systems establish a communication path across the accessing system from the subject's device to a network before communication between subject and associate can begin. Currently, the establishment or release of this path is reported via the BEGIN and END Intercept-Related Information (IRI) records, respectively.  These IRI records identify when an intercept subject has established or ended the ability to communicate over a communication path. After a communication path is established by a wireless accessing system between the subject device(s) and the network, the subject can communicate directly with an associate over the connecting path.

Currently, TS 33.108 does not provide support for the delivery of IRI for packet communications.

Discussion: Consideration should be given to prescribing interception of certain events at the 3G Gateway GPRS Support Node (GGSN).  Currently, TS 33.108 designates interception at the Serving GPRS Support Node (SGSN), and as a national option at the GGSN.  The following events are applicable to the 3G GGSN:

- PDP context activation

- PDP context deactivation

- Start of interception with PDP context active.

Reporting of packet activity IRI at the SGSN might require the SGSN to extract information present at the SGSN, but not typically used for routing.  Reporting of packet activity IRI might be better performed by the GGSN for the following reasons:

- For incoming packets (to be delivered to the MS), if all packet information is received over an aggregate "pipe" from the Internet Service Provider (ISP) (i.e., over a common IP address/Port Address combination), then the GGSN will need to look at the packet to determine which PDP context to route the packet over.  If the GGSN looks at the packet to do this determination, then the GGSN is uniquely positioned to provide the source/destination addresses and port information for all packets with respect to lawful interception.

- For incoming packets, even if unique pipes are used (one pipe per user to each ISP), then typically a router will associate a given address with a given physical interface.  If they do not verify that an IP address assigned to the user is the one signaled in the IP packet from the ISP to the wireless provider, the GGSN could send packets intended for another user to the subject.  This could happen because of an error in the ISP.

Once a PDP context has been established it is important for Law Enforcement Agencies (LEAs) to know the interception subject is sending or receiving packets and IRI associated with the packets.  Reporting of this type of activity is called Packet Activity Reporting.  When packet activity is detected by the network (i.e., a packet is sent by the interception subject or packet is being sent to the interception subject), this event needs to be reported.  However, a separate report for each detected packet is not necessary, but rather a summary report can be provided to the LEAs.  Therefore, Packet Activity Reporting can include the sending of IRI relevant to each instance with several of these instances being grouped together in one reported record. For each instance of packet activity detection, LEAs need to receive the following information:

- Source IP Address

- Destination IP Address

- Source Transport Protocol

- Destination Transport Protocol

- Source Transport Port Number

- Destination Transport Port Number

From the perspective of J-STD-025, there is a precedent for intercepting similar information (i.e., Calling/Called Party Number, Source/Destination IP Address and calling/called party subaddress information).  In some configurations, the calling/called party subaddress information could serve a similar purpose to port numbers.  The main routing function within the PSTN/ISDN is on E.164 numbers, but the end-to-end application may look at the subaddress information to decide how to further process the information transported (e.g., identify the application or session).

Requirement: LEAs require the delivery of IRI for the parties to a communication to ascertain origin, direction, destination, or termination information for each communication generated or received by the intercept subject. LEAs recognize that this information might be redundant with information delivered over the HI3 if a Title III court order is in effect.  LEAs require the delivery of IRI for packet activity regardless of whether the transmission is successful. That is, delivery of IRI is required for packets sent by the intercept subject regardless of whether the transmission is successful to the intended destination. Similarly, delivery of IRI is required for packets received by the intercepting system destined for delivery to the intercept subject regardless of whether the transmission is successful to the intercept subject MS.

LEAs require that the following specific information be reported separately from the content of communication to appropriately identify the parties and nature of the communication in association with an access path (i.e., PDP Context path):

a) Access Path ID (which correlates Network Address Information to Access Path events such as PDP Context Activation and Deactivation);

b) Protocol used by the Transport Layer (e.g., TCP, UDP);

c) Source IP address;

d) Source transport layer port;

e) Destination IP address;

f) Destination transport layer port.

Recommendation: The author recommends the addition of new triggers to TS 33.107 and TS 33.108 to report the packet activity event and related information. It is recommended that a new IRI record be created to support U.S. CALEA requirements in this area. However, the CONTINUE record can be expanded with a new trigger for this purpose. Parameter enhancements are required to support the reporting of event information. The new text proposed in this contribution assumes that a new IRI record is created to support reporting of this event.

- Proposed Changes to TS 33.107 to address Packet Activity Reporting

  - Changes to Clause 5.1.1 of TS 33.107 to address Packet Activity Events

Add the following dashed items within the first dashed list as follows:

    - default count threshold per intercept-subject (note 4)

    - default timer threshold for packet activity reporting per intercept-subject (note 4)

    - indication whether content of communication interception shall occur at the GGSN (see note 5)

NOTE 4:  As an option this parameter could be provide a common value for all intercept subjects and/or for all surveillances related to a given intercept subject.

NOTE 5:  If content of communication interception shall occur at the GGSN, then the SGSN shall not perform this interception.

  - Changes to Clause 5.1.2 of TS 33.107 to address Packet Activity Events

Add the following dashed items within the first dashed list as follows:

    - default count threshold per intercept-subject (see note)

    - default timer threshold for packet activity reporting per intercept-subject (see note)

NOTE: As an option this parameter could be provide a common value for all intercept subjects and/or for all surveillances related to a given intercept subject.

  - Changes to Clause 5.1.3 of TS 33.107 to address Packet Activity Events

Add a dashed item within the first dashed list as follows:

-	indication whether content of communication interception shall occur at the GGSN (see note)

NOTE:  If content of communication interception shall occur at the GGSN, then the SGSN shall not perform this interception.

  - Changes to Clause 7.3 of TS 33.107 to address Packet Activity Events

Change the first sentence as follows:

Intercept Related Information (Events) are necessary at the Mobile Station Attach, Mobile Station Detach, PDP Context Activation, Start of intercept with PDP context active, PDP Context Deactivation, RA update, Packet Activity, and SMS events.

  - Changes to Clause 7.3 of TS 33.107 to address Packet Activity Events

Change the first sentence as follows:

Intercept Related Information (Events) are necessary at the Mobile Station Attach, Mobile Station Detach, PDP Context Activation, Start of intercept with PDP context active, PDP Context Deactivation, RA update, Packet Activity, and SMS events.

  - Changes to Clause 7.3.1 of TS 33.107 to address Packet Activity and Roaming

Add the following as the new last paragraph:

In the case where the 3G GGSN is performing packet activity reporting, the user roams, and the 3G GGSN continues to be involved in the handling of the content of communications, the 3G GGSN shall continue to perform the packet activity reporting.

  - Changes to Clause 7.3.2 of TS 33.107 to address Packet Activity Events

- Change the first paragraph as follows:

There are seven eight different events in which the information is sent to the DF2 if this is required. Details are described in the following section. The events for interception are configurable (if they are sent to DF2) in the 3G GSN and can be suppressed in the DF2.

- Change the third paragraph as follows:

The following events are applicable to the 3G GGSN:

  - PDP context activation;

  - PDP context deactivation;

  - Start of interception with PDP context active;

  - Packet Activity.

- Add the following as a continuation of the third paragraph providing a list of events applicable to the 3G GGSN:

When Packet Activity reporting is performed at the GGSN, then content of communication interception shall also be performed at the GGSN and not at the SGSN.

  - Changes to Table 2 of TS 33.107

Event Type Description which type of event is delivered: MS attach, MS detach, PDP context activation, Start of intercept with PDP context active, PDP context deactivation, SMS, Cell and/or RA update, Packet Activity.

Packet Count Count of the number of packets detected over the packet activity reporting interval.

Packet Activity Report Reason The packet activity report reason indicates the reason for generation of the packet activity report

Originating Address Address of the source of the IP packet.

Destination Address Address of the destination of the IP packet.

Transport Protocol Information The transport protocol information includes the set of observed triplets (transport protocol [e.g., TCP], originating transport port number, and destination transport port number) over the packet activity reporting interval.

  - Add Clause 7.4.9 to TS 33.107 to address Packet Activity Reporting

The Packet Activity report event is generated at the 3G GGSN, when the GGSN has detected packet activity for the intercept subject; this event provides a summary of the intercept subject packets detected over a given time interval.  The fields will be delivered to the DF2, if available:

Observed MSISDN

Observed IMSI

Observed IMEI

PDP address of observed party

Event Type

Event Time

Event Date

Correlation number

Access Point Name

PDP Type

Network Element Identifier

Packet Count (if applicable)

Packet Activity Report Generation Reason (if applicable)

Originating Address (if applicable)

Destination Address (if applicable)

Transport Protocol Information (if applicable)

IAs (if applicable)

- Changes to TS 33.108 to address Packet Activity Reporting Events

  - Changes to Clause 6.5 of TS 33.108

The following text changes need to be incorporated into Clause 6.5 of TS 33.108:

There are eight nine different event types received at DF2 level. According to each event, a Record is sent to the LEMF if this is required. The following table gives the mapping between event type received at DF2 level and record type sent to the LEMF.

- Table 1: Mapping between UMTS Data Events and HI2 records type

| Event | IRI Record Type |
| --- | --- |
| GPRS attach | REPORT |
| GPRS detach | REPORT |
| PDP context activation (successful) | BEGIN |
| PDP context activation (unsuccessful) | REPORT |
| Start of intercept with PDP context active | BEGIN |
| PDP context deactivation | END |
| Cell and /or RA update | REPORT if no PDP context is active CONTINUE if, at least, one PDP context is active |
| SMS | REPORT |
| Packet Activity | PACKETACTIVITY |

  - Additions to Table 2 of TS 33.108

The following additional parameters need to be added to Table 2 of TS 33.108:

- Table 2: Mapping between Events information and IRI information

| parameter | description | HI2 ASN.1 parameter |
| --- | --- | --- |
| Originating Address | Reports the source address in an IP packet when reporting packet activity. | originatingAddress |
| Destination Address | Reports the destination address in an IP packet when reporting packet activity. | destinationAddresss |
| Packet Activity Count | Reports the number of packets detected over a given monitoring reporting interval. | packetActivityCount |
| Packet Report Reason | Reports the reason for the generation of the packet activity report record. | packetActivityReportReason |
| Originating Port Number | Reports the source transport layer port in an IP packet when reporting packet activity. | packetActivityProtocolInfo (originatingPortNumber) |
| Destination Port Number | Reports the destination transport layer port in an IP packet when reporting packet activity. | packetActivityProtocolInfo (destinationPortNumber) |
| Transport Protocol | Reports the Protocol used by the Transport Layer for the source in an IP packet (e.g., TCP, UDP). | packetActivityProtocolInfo (transportProtocol) |

  - Additions to Clause 6.5 of TS 33.108

Add a new clause to TS 33.108, namely Clause 6.5.1.5, with the following proposed text.

#### 6.5.1.5	PACKETACTIVITY record information

The PACKETACTIVITY record is used to convey IRI for the parties to packet communications after a communication path is established by a wireless accessing system between the subject device(s) and the network.

The PACKETACTIVITY record shall be triggered when:

- the intercept subject generates or receives IP packets, and the Packet Activity Count for each originatingAddress/destinationAddress combination reaches or exceeds the Packet Activity count threshold value provisioned for the interception subject lawful interception record; or

- the intercept subject generates or receives IP packets, and the time threshold value provisioned for the interception subject lawful interception record is reached or exceeded.

A PACKETACTIVITY record can optionally be triggered when:

- the intercept subject generates or receives IP packets, and the originatingAddress / destinationAddress combination changes between consecutive packets generated or received by the intercept subject.

Table 6-16: Parameters for Reporting Packet Activity Events

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| observed IMEI |  |  |
| observed PDP address | C | Provide to identify the PDP address of the intercept subject. |
| event type | C | Provide Packet Activity event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| Access Point Name | C | Provide to identify the packet data network to which the intercept subject is connected. |
| PDP type | C | Provide to describe the PDP type of the observed PDP address. |
| network identifier | M | Shall be provided. |
| correlation number | C | Provide to uniquely identify the PDP context delivered to the LEMF and to correlate IRI records with CC. |
| lawful intercept identifier | M | Shall be provided. |
| GGSN address | C | Provide to identify the GGSN node. |
| Packet Report Reason | M | Provide to identify the reason for the generation of the packet activity report (e.g., end of default time interval). |
| Originating Address | M | Provide to report the source address in an IP packet when reporting packet activity. |
| Destination Address | M | Provide to report the destination address in an IP packet when reporting packet activity. |
| Packet Activity Count | M | Provide to identify the number of packets detected with the same originatingAddress/destinationAddress combination. |
| Packet Activity Protocol Info | M | Provide the set of observed triplets (transport protocol [e.g., TCP], originating transport port number, and destination transport port number) over the packet activity reporting interval and associated with the same originatingAddress/destinationAddress combination. |

To support the Packet Activity event, the GGSN shall be able to intercept individual packets sent from or to the interception subject.  The interception of each such packet and the retrieval of the required information from that packet can be considered an event.  However, this particular event (i.e., each individual intercepted packet) shall not be reported individually, rather the required information from each packet shall be reported via the Packet Activity event.

A count of the number of intercepted packets with a given originating/destination address combination is maintained provide input to a common PacketActivity record.  If this count reaches or exceeds a provisioned threshold for the intercept subject, then a packet activity record is generated.  The range of the count threshold is 500 to 5000 with a default value of 2500.

A packet activity monitoring interval is utilized to govern the amount of time over which intercepted packets with a given originating/destination address combination provide input to a common Packet Activity report.  If this monitoring interval reaches or exceeds a provisioned threshold for the intercept subject, then a PacketActivity record is generated.  The range of the timer threshold is 30 seconds to 5 minutes in (30 second increments) with a default value of 1 minute.

  - Additions to Clause B.4 of TS 33.108

The ASN.1 encoding in Clause B.4 of TS 33.108 must be modified to allow for the inclusion of the definition of the PACKETACTIVITY record (additions are underlined).

IRIContent		::= CHOICE

{

iRI-Begin-record		[1] IRI-Parameters,

--at least one optional parameter must be included within the iRI-Begin-Record

iRI-End-record			[2] IRI-Parameters,

iRI-Continue-record		[3] IRI-Parameters,

--at least one optional parameter must be included within the iRI-Continue-Record

iRI-Report-record		[4] IRI-Parameters,

--at least one optional parameter must be included within the iRI-Report-Record

iRI-PacketActivity-record		[5] IRI-Parameters,

--at least one optional parameter must be included within the

--iRI-PacketActivity-Record

...

}

The following five IRI-parameters need to be added to the ASN.1 encoding in TS 33.108 for reporting the Packet Activity events.

originatingAddress		[26] IPAddress	OPTIONAL,

-- The sourceAddress parameter contains the

-- source ip address in the intercepted ip packet.

destinationAddress		[27] IPAddress	OPTIONAL,

-- The destinationAddress parameter contains the

-- destination ip address in the intercepted ip packet.

packetActivityCount	[28] Count OPTIONAL,

-- The packetactivityCount parameter reports the number of packets

-- detected that utilized the same originatingAddress/terminatingAddress

-- combination over a given packet activity interval

packetActivityReportReason [29] ReportReason OPTIONAL,

-- The packetActivityReason parameter indicates the reason for generating the

-- Packet Activity Report record.

PacketActivityProtocolInfo [30] PacketActivityProtocolInfo OPTIONAL,

The following four parameter definitions need to be added to the ASN.1 encoding in Clause B.4 of TS 33.108 for reporting the Packet Activity events.

-- PARAMETERS FORMATS

PacketActivityProtocolInfo ::= SEQUENCE OF

{

transportProtocol	[0] TransportProtocol OPTIONAL,

-- The transportProtocol parameter reports the

-- Transport Protocol used by the Transport Layer(e.g., TCP, UDP, SCTP)

-- in an IP packet when reporting packet activity. The

-- transportProtocol parameter shall be coded in accordance

-- with document reference [51].

originatingPortNumber		[1] PortNumber	OPTIONAL,

-- The originatingPortNumber is the source transport layer port

-- in an IP packet when reporting packet activity.  The

-- originatingPortNumber parameter shall be coded in accordance with

-- document references [52], [UDP], and [SCTP].

destinationNumber		[2] PortNumber	OPTIONAL,

-- The destinationPortNumber is the destination transport layer port

-- in an IP packet when reporting packet activity.  The

-- destinationPortNumber parameter shall be coded in accordance with

-- document references [52], [UDP], and [SCTP].

}

TransportProtocol::= OCTET STRING (SIZE (1))

-- From “Assigned Numbers” RFC 1700 available from the IETF or more recent information

-- can be found at http://www.iana.org/assignments/protocol-numbers.

PortNumber ::= INTEGER (0..65535)

Count ::= INTEGER (0..65535)

ReportReason ::= ENUMERATED

{

timerExpiry (0),

countThreshold (1),

addressChange (2),

…

}

The ASN.1 encoding of the GPRSEvent parameter in TS 33.108 needs to be modified to add a codepoint for this event (See Clause B.2.6). Within the ASN.1, the following value should be added

GPRSEvent ::= ENUMERATED

{

pDPContextActivation 					(1),

startOfInterceptionWithPDPContextActive	(2),

pDPContextDeactivation					(4),

gPRSAttach 								(5),

gPRSDetach 								(6),

locationInfoUpdate 						(10),

sMS 									(11),

packetActivity							(12?),

...

}

-- see ref [10]

C. Recommendation

Approve all recommended changes.

STD0006 " User Datagram Protocol (UDP)", 28 August 1980.

RFC 2960 " Stream Control Transmission Protocol (SCTP)", October 2000.