|  | ANGLO-AUSTRALIAN OBSERVATORY P.O. BOX 296 EPPING N.S.W. 2121 AUSTRALIA Telephone: +61 2 9372 4800 Fax: +61 2 9372 4880 |
| --- | --- |

FPOSS User Manual

| Document Number: | INS-MAN-AUS-13271-0079 |
| --- | --- |
| Issue Number: | 1.6 |
| Issue Date: | 2 Dec 2002 |

| Prepared: | Keith Shortridge | 2 Dec 2002 |  |
| --- | --- | --- | --- |
|  | Name | Date | Signature |
| Approved: | Tony Farrell | 2 Dec 2002 |  |
|  | Name | Date | Signature |
| Released: | Chris Evans | 2 Dec 2002 |  |
|  | Name | Date | Signature |

- Change Record

| Issue / Revision | Date | Section / Page affected | Reason / Remarks |
| --- | --- | --- | --- |
| 1.0 | 29/09/1999 | All | First Issue - preliminary |
| 1.1 | 25/09/2001 | All | PAA version |
| 1.2 | 25/11/2001 | Most | PAA(B) preliminary version |
| 1.3 | 14/02/2002 | Most | First released version |
| 1.4 | 14/02/2002 | 3.8,6.14,9.7 | Minor changes relating to PAF file names |
| 1.5 | 30/08/2002 | 7 | Updated distribution details |
| 1.6 | 02/12/2002 | 5, figures | File format modified, up-to-date figures used and some descriptive text for figures updated. |

FPOSS User Manual

- Contents

1	Introduction	5

1.1	Purpose	5

1.2	Scope	5

1.3	Reference Documents	5

1.4	Abbreviations and Acronyms	5

1.5	Nomenclature	6

1.6	Glossary	6

2	Overview	7

3	Using FPOSS	7

3.1	Initial preparations	7

3.2	Preparing the input file	9

3.3	Running FPOSS	10

3.4	Reading in a target file	10

3.5	Selecting a VLT guide star	10

3.6	Specify the fibre combination	11

3.7	Perform the allocation	11

3.8	Saving the configuration	11

3.9	Exit the program	12

3.10	Additional checks	12

4	Some additional features	13

4.1	Using the Mimic window	13

4.2	Different allocation algorithms	13

4.3	Uncrossing fibres	13

4.4	Multiple configurations to cover a target list	13

4.5	Viewing allocations using Skycat	14

4.6	Reference	15

5	Detailed input file format	16

5.1	Additional items embedded in comments	17

5.2	Possible object types:	17

5.3	Example Text Input File	18

6	The FPOSS configuration program	19

6.1	Running the FLAMES configuration program	19

6.2	Opening files	19

6.3	Converting Files	20

6.4	Setting the Wavelength	20

6.5	Setting the Field Plate	20

6.6	Overview of the Allocation Procedure	21

6.7	The Allocation Procedure in detail	21

6.7.1	The ‘Oxford’ algorithm	22

6.7.2	The ‘Taylor’ algorithm	23

6.7.3	Tentative deallocation	24

6.7.4	Fibre uncrossing	24

6.7.5	Sky allocations	24

6.8	Target Priorities	25

6.9	The allocation window	25

6.10	Sky Fibres	25

6.11	Manual Allocation of Fibres	26

6.12	The Mimic Display	27

6.13	Saving a Configuration	27

6.14	Batch Mode	28

6.15	Command Line Switches	28

6.16	Reference	29

7	Obtaining and installing the program	30

8	The Menus in detail	31

8.1	The File menu	31

8.2	The View menu	32

8.3	The Options Menu	32

8.4	The Commands Menu	33

8.5	The Zoom menu	35

9	Appendix A: A tour of  FPOSS	36

9.1	Starting up	36

9.2	Setting a magnitude filter	38

9.3	Opening the data file	40

9.4	Selecting the VLT guide star	43

9.5	Selecting the Fibre combination	46

9.6	Performing the Allocation	48

9.7	Saving the allocation	49

9.8	Things not in the basic sequence	50

9.8.1	Changing the central wavelength	51

9.8.2	Checking Allocation Validity	51

FPOSS User Manual

# Introduction

This manual describes the FPOSS Fibre configuration program written at AAO as part of the FLAMES project. FPOSS is intended to be run as an adjunct to the P2PP process for FLAMES observations. It takes as input a file containing a list of target objects and generates a configuration in which as many fibres as possible are allocated to targets, allowing for the various instrumental constraints and any specified target priorities. It produces a file containing a list of allocations of fibres to targets. This file needs to be included as part of the observation description that results from the P2PP process.

FPOSS is run as a standalone program on the observer’s own computer system. It is available from ESO in versions suitable for Linux, HP-UX and Solaris.

This manual begins with a brief summary of how to use the program, aimed at FLAMES observers. This is followed by a detailed formal description of the input text file format. The section after that presents a more formal description of the configuration program.

An appendix contains an introductory tour of the program, taking you in detail through an allocation from the reading of a supplied input text file to the writing of the output PAF file. You may like to run through this when you start to use the program, just to get familiar with the way it works. The appendix provides little that cannot be found in the main body of the manual, apart from a number of illustrations, but some users may find this the most convenient introduction to FPOSS.

## Purpose

This document serves as the User Manual of the FLAMES fibre configuration software. It has been written for operators of the instrument and astronomers. The reader is assumed to be familiar with the FLAMES instrument.

## Scope

This manual only covers the operation of the FPOSS program itself. It does not cover the FLAMES P2PP process, nor does it cover any of the details of observing with FLAMES.

## Reference Documents

The following documents are referenced in this document:

- INS-INS-AUS-13721-0017, 4 17/11/1997 -- Flames Fibre Positioner, Glossary of Terms

- VLT-MAN-ESO-19200-1644/2.1B – P2PP User’s Manual

## Abbreviations and Acronyms

The following abbreviations and acronyms are referenced in this document:

| 2dF | The AAO’s 2 degree Field Facility. |
| --- | --- |
| AAO | Anglo-Australian Observatory |
| ARGUS | Integral Field Unit (fixed) |
| BOB | Broker for Observation Blocks |
| FACB | Field Acquisition Coherent Bundle |
| FLAMES | The Whole Fibre Facility |
| FP | Fibre Positioner |
| FPOSS | The program described in this document. |
| GUI | Graphical User Interface |
| OS | Observation Software |
| OSS | Observer Support Software |
| PAF | Standard file format for VLT internal files |
| TCL | Tool Command Language |
| TK | Tcl Windowing Toolkit. |
| VLT | Very Large Telescope |

## Nomenclature

Throughout this manual the term ‘reference star’ is used for a target that is intended to be allocated to the FACB guide fibre bundles. The term ‘guide star’ is only used for a star intended as a target for the VLT guide probe. Earlier versions of this manual, and of the program, used the terms ‘reference star’, ‘guide star’ and ‘fiducial’ more or less interchangeably.

## Glossary

A glossary of terms specific to the Fibre Positioner is given in the separate document Flames Fibre Positioner, Glossary of Terms [1]

# Overview

The configuration program is part of the observation preparation software used by a number of instruments built by the AAO. These include the original '2dF', the more recent '6dF' and the 'OzPoz' fibre positioner built for the 'FLAMES' instrument for ESO. A version of this program is distributed by ESO as an adjunct to the P2PP process for FLAMES, and this version of the program is known as FPOSS (Fibre Positioner Observation Supoort Software). All of these instruments use a robotic fibre positioner to place optical fibres in the focal plane of the telescope so that light from a number of target objects may be fed to spectrographs or other instruments. These all need a configuration phase in which the fibres are allocated to targets in such a way that the fibre configuration is physically possible (fibres and the 'buttons' that carry them do not collide, and fibres are not bent past their allowed tolerances, for example), and the best coverage possible is achieved (as few unallocated targets as possible, without any strange artifacts in the distribution of allocated targets, for example).

The role of the configuration program (FPOSS) is to take a text file containing a list of target objects and to allocate fibres to these in as optimal a way as possible. It then writes out a file giving the details of the resulting configuration. When FLAMES is being configured, this file is written out in ESO’s parameter file format (PAF), and the name of the file in question has to be included in the observation block created as part of the P2PP process.

The rest of the manual is organised as follows:

- Chapter 3 summarises the use of FPOSS for FLAMES observers.

- Chapter 4 describes some more advanced features of the program.

- Chapter 5 provides information about file formats.

- Chapter 6 provides comprehensive descriptions of program facilities.

- Chapter 7 gives details about obtaining and installing the software.

- Chapter 8 provides menu guides and explanations.

- Chapter 9 is an appendix that provides an illustrated tour of FPOSS.

# Using FPOSS

This section provides a description of how FLAMES observers should use FPOSS to prepare configuration files. This section attempts to be brief; more details can be found in the rest of the manual. If this section seems too brief, or if you have problems running the program with your own files, you should try following the detailed tour in chapter 9 in order to become familiar with the program.

## Initial preparations

You need to know the following things:

- Where your target field is. You need to know:

A central position in J200 coordinates.

A name for the field.

The name can be anything you choose. You can modify the field center position slightly in FPOSS, but you need a good initial position.

- When you intend to observe it.

You need to specify a day. You probably don’t know this exactly, but you can make a reasonable guess.

- What FLAMES configuration(s) you intend to use.

You need to know which combination of fibres you will use for each observation. You need to know if you will use UVES, ARGUS or IFU or Medusa fibres, and in what combination. If ARGUS is to be used, you need to know the position angle to be used.

- What targets you want to observe. You need a list of targets, and for each one you need:

A name.

The position in J2000 coordinates.

The magnitude.

The target type.

A priority.

The magnitudes are recorded in the output files, but are otherwise ignored except for VLT guide probe targets, in which case the magnitude should be within a specified range. Targets of high priority are allocated to fibres in preference to ones of lower priority. Priorities are classified as a numeric code from 0 to 9, with 9 the highest priority. The target type determines what fibre types can be assigned to the target, so you need to know which targets are to be observed with which instrument.

- The locations of some possible VLT guide stars

You need to supply the position of at least one star suitable for use as a VLT guide star. Ideally, these should be close to the edge of the field in order to reduce the shadowing of the field by the VLT guide probe. These need to have magnitudes suitable for use by the VLT guider. You will choose one of these during the configuration process.

- The locations of some reference stars

You also need to supply a number of stars that can be used as reference objects by the FACB guide fibres. At least three of these need to be included in the configuration, so you should supply a number of possible reference stars, arranged throughout the field.

- Some sky positions

You can handle sky in more than one way, but the most control over sky is obtained by providing explicit sky targets – coordinates that refer to blank sky – in the target list. You can specify that sky targets are only for use by the IFU sky fibres (you might place these close to the IFU targets), or only for use by the ARGUS sky fibres (you might place these close to the main ARGUS aperture in the center of the field), or general sky targets that can be used for any instrument. Alternatively, FPOSS can generate a ‘sky grid’ of possible sky positions, but it cannot guarantee that these actually represent blank sky. However you allocate sky targets, you should ensure that your configurations have enough sky targets allocated to fibres.

## Preparing the input file

Most the information listed in the previous section needs to be supplied to FPOSS in a text file. Such files are usually called ‘.fld’ files, since that is the usual extension that they use. These can be created using an ordinary editor, but it is more likely that the bulk of the file will be created using a program. The format of the file is detailed in section 5 but it has a short initial section describing the field itself and then a series of lines each one of which describes one target object. (Some of which describe reference stars and some of which describe potential VLT guide stars.)

The field details at the start of the file might look like this:

LABEL M67 central field

UTDATE 2001 04 23

CENTRE 08 51 22.82  11 50 09.4

And the following might be lines describing individual targets:

M67_00005       08 50 15.98   11 33 58.6   M  1  18.16      1  (B-V)= 0.614

M67_00050       08 51 47.93   11 34 45.6   U  1  15.33     19  (B-V)= 0.321

M67_00097       08 50 38.40   11 35 32.2   I  1  18.06     42  (B-V)= 0.103

U0975_06059928  08 50 18.307  11 55 21.29  G  1   9.40   1052

U0975_06066823  08 51 23.004  11 49 13.01  F  1  12.30   1093

SKY_00023       08 51 54.80   11 45 07.8   S  1   99.9   1338

ISKY_0023       08 51 09.81   11 52 14.1   J  1   99.9   1373

ASKY_0001       08 51 10.87   11 42 47.9   A  1   99.9   1374

Each of these sample lines describes a target of a different type, distinguished by the single letter target type code that follows the coordinates. ‘M’ is a Medusa target, ‘U’ a UVES target, ‘I’ is an IFU target, ‘J’ is an IFU sky target, ‘G’ is a potential VLT guide star, ‘A’ is an ARGUS sky target and ‘F’ is a reference star target (a ‘fiducial star’) for the FACB guide fibre bundles.

It is possible to generate a number of configurations from the same input file. If you intend to observe the same field with different combinations of fibre types you can include the targets for all fibre types in the one file and generate different configurations with different fibre types enabled. If you have a large number of targets in the same field you can put them all in the one file and generate a number of configurations until all the targets have been allocated.

## Running FPOSS

You obtain a copy of the various files needed by FPOSS from ESO, and install them in a directory on your local machine. Where you choose to do that is up to you. For simplicity here, we assume the directory is in your execution path, so just typing the name of an executable file in it will run that program. The FPOSS executable is called ‘configure’. The most convenient way to run it is probably to have a working directory in which you have your .fld files and in which you will generate your configuration files. With your default set to that working directory, type

configure &

This will start up FPOSS. There is a standard basic sequence that will guide you through the steps needed for a standard configuration. The basic sequence window is shown in Figure 6, and a configuration can be created and saved simply by clicking its buttons in order. (The tour through FPOSS in the appendix - section 9 - shows the user interface in detail as one follows this sequence.) Essentially, you have to do the following:

- Set a magnitude filter to reduce the number of targets read from the .fld file, if necessary.

- Read in the details of a target field from the .fld file.

- Select a VLT guide star.

- Specify the fibre combination to be configured.

- Perform the allocation of fibres to targets.

- Save the resulting configuration in a PAF file.

Each of these steps corresponds to one button in the basic sequence window.

## Reading in a target file

The program has no preset limit to the number of targets it can handle, but in practice it starts to run slowly (particularly when displaying) if it is dealing with more than a couple of thousand targets. If you have more targets than that, you should apply a magnitude filter so that only targets in the same magnitude band are handled at one time. (The filter does not apply to reference stars and VLT guide stars.) To apply a magnitude filter, click on the ‘magnitude filter’ button in the basic sequence window and enter a pair of limiting magnitudes. Then click on ‘apply’.

To read in a target file, click on the ‘open file’ button in the basic sequence window. Then select the file (usually a .fld file) in the file selection dialogue. The field will be displayed, together with all the fibres.

## Selecting a VLT guide star

Potential VLT guide stars are shown in dark blue. You should select one by double-clicking on it. You then see two blue circles that indicate the two possible pivot positions for that guide star. Click on one of these to select that position. Ideally, the guide star should be selected close to the edge of the field so as to mimimise the shadowing by the guide probe.

(Usually the guide stars can be seen clearly. If you cannot see them, go to the main control panel and select ‘Select targets’ from the ‘View’ menu. Click the boxes for ‘VLT guide’ and

‘highlight using arrows’ and click on the ‘apply’ button.)

Note: The ‘Select guide star’ button in the basic sequence window merely reminds you how to do this and records that you have done so; it doesn’t actually initiate the selection for you, since all you usually have to do is double click on a suitable target.

## Specify the fibre combination

Click on ‘Fibre combination’ in the basic sequence window. A list of possible fibre combinations appears. You should select the one you want to use and click ‘apply’. Note that the FACB reference fibres are included in all combinations and do not need to be specified explicitly.

## Perform the allocation

Click on the ‘allocate’ button in the basic sequence window. A dialogue window appears offering you various options that control the details of the allocation process. Usually, the default values will be fine, and all you need to do is click the ‘OK’ button. You should check the number of sky targets that are to be allocated. If you are using fibres that can be allocated to either sky or non-sky targets it is usually best to tell the program explicitly to allocate a certain number of sky targets to each fibre type. If you have too few sky fibres allocated you may not be able to get the best out of your data when you reduce it.

Section 6.9 provides more details of the allocation process and explains the various options.

The total number of fibres allocated is shown in the basic sequence window. More details, including the number allocated to reference stars and to sky targets are shown in the control panel. You should check that you have enough sky are reference targets allocated to fibres.

If the allocation is unsatisfactory for any reason, you can try again with different parameters (in which case you should select ‘remove allocations’ from the ‘commands’ menu first). Or you can make minor corrections to the allocation by hand, selecting targets and fibres by clicking on them and either explicitly allocating and deallocating them. This is described in more detail in other parts of this manual.

## Saving the configuration

Click on the ‘Save as PAF’ file button in the basic sequence window. A file save dialogue will allow you to specify the name and location of the file to be written. The name of the file is generated automatically according to FLAMES conventions, and should not normally be changed. This file is a text file and can be printed or examined, but it should not be modified. Changes to the output PAF file, no matter how small,  will almost certainly cause it to be rejected by the observing system when the time comes to make use of it.

You should also be careful not to introduce changes to the PAF file inadvertently. Some e-mail systems (Netscape is an example) feel entitled to modify text files in subtle ways. If you need to transfer a PAF file you should gzip it first, then transfer the gzipped file and gunzip it on the receiving machine.

## Exit the program

Click on the ‘Exit’ button in the basic sequence window to exit the program.

## Additional checks

The basic sequence described above will usually generate perfectly satisfactory configurations. However, you may want to perform some additional checks yourself.

Changes in central wavelength and hour angle introduce small changes in the postion of target objects in the field. FPOSS configures its fields for a specific hour angle and central wavelength. If the actual observation is made under radically different circumstances, the target positions will change and in some circumstances these changes can result in two fibres being positioned too close together, or the maximum bend angle for a fibre being exceeded. FPOSS allocates using more restrictive tolerances for such things as button/fibre clearances and maximum fibre bend angles than does the actual positioner. This means that configurations generated by FPOSS should be valid over a wide range of hour angles and various central wavelength values. You may wish to test this. You can change the central wavelength using the ‘Set Wavelength’ option in the ‘Commands’ menu, and can then check the validity of your fibre allocation at this new wavelength using the ‘Check Allocation’ option in the ‘Commands’ menu. You can check the validity of the allocation over a range of hour angles using the ‘Check over HA range’ option in the ‘Commands’ menu.

# Some additional features

This section describes in a relatively informal way some of the additional features of the program.

## Using the Mimic window

A number of features are available as you move the cursor around the mimic window. Some of the more useful are as follows:

You can zoom in and out using the ‘zoom’ menu in the main panel, but you can also zoom in by a factor two by clicking the right mouse button in the mimic window. You can zoom out by a factor two by control-clicking the right mouse button in the mimic window.

As the cursor moves over a fibre, the fibre number and type are displayed at the top information pane of the screen. If you click on a fibre, this selects the fibre.  As the cursor moves over a target, the target name and type are displayed at the top of the screen. If you click on an unallocated target, this selects the target. (It’s hard to click on an allocated target, because the fibre covers it and you select the fibre instead. In practice, however, you rarely want to select an allocated target – selecting the fibre is usually what you want to do.)

If you have selected an unallocated fibre and an unallocated target, you can use the ‘allocate target’ option in the ‘commands’ menu to allocate the selected fibre to the selected target. If you have selected an allocated fibre, you can use the ‘deallocate fibre’ option in the ‘commands’ menu to deallocate it. This allows you to impose your own choice of allocations if you really want to do so.

## Different allocation algorithms

The program supports a couple of different allocation algorithms, and these can all be run in a number of different ways. These are controlled by the options presented to the use by the dialogue box displayed when the ‘Allocate…’ option is selected. These are described in detail later in this document.

## Uncrossing fibres

It is possible to explicitly invoke a version of the allocation algorithm than only attempts to reassign already allocated fibres in order to reduce the number of crossovers of fibres. This can be done by selecting ‘uncross fibres…’ from the Commands menu. The fewer crossovers there are in a configuration the faster it is for the fibre positioner to reconfigure the field.

## Multiple configurations to cover a target list

If you have a large number of targets – such as those in the sample input file N3201-00.fld – then you may want to try to create a number of configurations that will cover all of the targets. The program is not optimised for this mode of operation (this is something that needs discussion with ESO), but it can be used in this mode.

After a first configuration, which will, of course, only have allocated fibres to a small subset of the targets available, select the ‘List…’ option from the ‘File’ menu. A dialogue box will allow you to control what is output to the list file. Select the ‘unallocated objects’ option. This will produce a new target list that contains only the unallocated targets and the reference targets. (All guide fibres are included, even those allocated in the present configuration, since they can sensibly be reused in the next configuration.). You are then prompted for the name and location of the new list file.

You can then select the ‘Open…’ option from the ‘File’ menu and specify this new target list as the input file. You now have all the targets except for those non-guide targets allocated in the previous configuration. You can create a new allocation and repeat the process.

(This will get tedious if exhaustive coverage is needed – i.e. if the intention is to cover all the targets in the input list – and the final configurations will have relatively few targets. The tediousness can be addressed by automating the procedure if this is going to be useful. These are the aspects that need further discussion based on experience with the interim version of the program.)

## Viewing allocations using Skycat

You may want to see the allocated targets plotted against an image of the sky. In particular, if you have sky targets you may want to check on whether or not these really do represent blank sky. FPOSS can generate catalogue files in the standard ESO catalogue format containing all the allocated targets, and these can be read by the ESO SKYCAT program.

To do this, select the ‘List…’ option from the ‘File’ menu, and select the ‘Allocated objects in Skycat format’ option from the dialogue box that comes up. You are then prompted for the name of the catalogue file to be written.

If you now run Skycat, you can read this newly created catalogue file as a local catalogue file.  (Use the menu sequence Data-Servers -> Local Catalogs -> Load from file.) If you do so immediately after running Skycat, with no image displayed, Skycat will default to a display centered on the center of the field that was allocated, and will show some of the allocated targets around the center of the field. To get the allocated objects displayed on a sky image, use the ‘Image servers’ sub-menu.  (The usual menu sequence is: Data-Servers -> Image Servers -> Digitized Sky at ESO.) The center of the field will already be set, but you should increase the size of the field to be somewhere in the range 25 to 30 arcmin, depending on how much of the field you want to see. An image will be retrieved from the image server, and will be displayed with the allocated targets superimposed on it.

By default, sky targets will be displayed as white squares, reference stars (FACB targets) will be displayed as white diamonds, and all other targets will be displayed as white circles. Skycat experts can change these defaults. The catalogue as written by FPOSS contains a text entry that describes the target type precisely, and numeric entries that indicate whether or not each target is a sky target or a reference target. There is one dummy target included in the catalogue, the first one listed, that indicates the center of the field.

## Reference

P2PP User’s Manual  - David Silva, ESO Support Group

VLT-MAN-ESO-19200-1644/2.1B

# Detailed input file format

This section refers to the text file that is used the first time a set of targets are presented to FPOSS.  Usually these are files with a .fld extension, although that is not necessary. These files are prepared by the astronomer – possibly using a purpose-built program to do so.

The file consists of lines of up to 256 characters. Comment lines can be indicated by an asterisk character  (‘*’) or a hash character (‘#’) in the first column. At the start of the file, there should be a series of lines that describe the field itself, as opposed to the individual targets it contains. Special characters, particularly quotes, should be avoided. Each of these lines must begin with one of the following keywords (which may appear in any order):

LABEL	A string giving the target field label.

UTDATE	The UT Date of observation.

CENTRE	Field Centre R.A. and Dec. (hh mm ss.ss -dd mm ss.s)

EQUINOX	Coordinate equinox – must be J2000.0

ARGUS	Argus angle and scale values.

Of these, only LABEL and CENTRE are required.

Any non-comment line that does not start with one of these keywords will be assumed to signal the start of the target list.  The ARGUS line is optional, and is used only to provide an initial value for the ARGUS scale and position angle. The equinox of all coordinates given in the file must be J2000.0 for FPOSS. Although the EQUINOX line is allowed for FPOSS it is optional and if included the value specified must be J2000.0 – any other value will be rejected.

LABEL can be followed by any text. It does not have to be enclosed in quotes.

UTDATE must be followed by a date in the format yyyy mm dd[.dd]. The fractional date shown here as [.dd] indicates that a time of day can be included as a decimal fraction of a day. UTDATE is used only because FPOSS needs a date and time in order to produce a configuration. If no UTDATE line is supplied, FPOSS will use the current date – since it produces configurations that will remain valid over a large range of dates and times there is no particular reason to supply a date, and at P2PP time the date of observation will not normally be known. If no time of day is specified it will choose a time of day that puts the field on the meridian for the supplied date.

CENTRE must be followed by an RA and Dec in the format hh mm ss.ss [-]dd mm ss.s. The [-] indicates an optional sign.

EQUINOX, if supplied, must be followed by the string J2000.0. For FLAMES the use of J2000.0 is mandatory – other instruments allow other equinox values to be specified.

ARGUS, if supplied, must be followed by a position angle in degrees and a string specifying the ARGUS scale. The scale string must be either ‘1:1’ or ‘1:1.67’ (the quote characters are not required).

Subsequent lines describe target objects, one per line. Each line consists of a number of items separated by spaces. The first four items are mandatory as follows:

Name 	The name of the object (by convention F<n>  is used for reference stars) Note that this cannot contain spaces.

RA 	(hh mm ss.ss)

Dec 	(-dd mm ss.s)

Type	One character indicating the type of object. The allowed types are listed below.

The program regards the following items as optional, and will not object if they are omitted. However, for FLAMES observing they should be provided. Any number may be used for the Program Id field. If you are not using priorities you should set all priorities to the same value, say 1. Any items not required must be filled in with an asterisk place-holder if any subsequent items are to be used.

Priority 	(1-9) with 9 being the highest priority

Magnitude 	(mm.mm)

Program Id	An integer which uniquely identifies a specific project. At present, this is ignored for FLAMES.

Comment 	Any remaining text up to the end of the line

## Additional items embedded in comments

FPOSS allows a number of optional additional items of information about target objects to be included in the input file. These are included in the comment field, and take the form of strings

(Name)=value

or

(Name=value)

These can be delimited by commas, blanks, or semi-colons, and should be at the start of the comment string. The following names are accepted within the parentheses.

BAND	Band designation for the magnitude specified (B,V,R)

B-V	Object colour index, B-V

V-R	Object colour index V-R

PM-RA	Annual right ascension proper motion for the equinox J2000 in the FK5 system in arcsec per year.

PM-DEC	Annual declination proper motion for the equinox J2000 in the FK5 system in arcsec per year.

SNR	Signal to noise ratio requested by the observer.

CATEGORY	Object morphology: Stellar or Extended

DIAMETER	Source extension diameter for extended objects, in arcmin.

## Possible object types:

The following letters are accepted in the ‘Type’ field for an object:

M	Program object. For FLAMES, this indicates a target that should be assigned to a single fibre (as opposed to a fibre bundle such as an IFU). This designation should be used for targets to be assigned to Medusa fibres from Giraffe.

I	IFU. This indicates a target that should be assigned to a GIRAFFE integrated field unit (IFU).

U	UVES. A target that should be assigned to a UVES fibre.

A	A sky target to be assigned to one of the Argus sky fibres.

J	A sky target to be assigned to an integerated field unit (IFU). This can only be assigned to an IFU sky fibre.

S	Sky. This indicates a position that can be used for sky. Such a target may be assigned to any Medusa fibre, to an IFU sky fibre, to an ARGUS sky fibre, or to a UVES fibre.

F	Fiducial (reference star). This indicates a target that should be assigned to a guide fibre – one of the FACBs.

G	Guide star. This indicates a guide star that can be used by the VLT guide probe. Fibres will not be assigned to such an object.

## Example Text Input File

* This is a comment line

LABEL target field number 1 xyz cluster

CENTRE 12 43 23.30 +10 34 10.0

* end of required header info

*

F1   12 40 20.55 +10 30 11.4    F   6  12.0  1  brightest star

F2   12 38 10.31 +09 59 58.9    f   6  13.5  1  fiducial star

*

NGC1002  12 41 30.55 +10 31 56.9  P 9  15.0  1  small fuzzy galaxy

ic3082   12 40 18.40 +10 32 21.5  P 9  *     1  candidate satellite

*

sky-1   12 40 10.00 +10 32 21.5   S 3  *     1  blank sky (checked)

# The FPOSS configuration program

This section is a more formal description of the OSS configuration program. It describes a number of aspects of the program that may not be relevant to ESO users. ESO does not expect FPOSS to be run in batch mode, for example.

The FLAMES configuration program is used to perform the following main functions:

- Performing the allocation of objects to fibres in a FLAMES field.

- Checking the validity of fibre allocations over a range of dates and hour angles.

- Generating a file describing the field configuration and allocations that can be used as input to the FLAMES observing system.

## Running the FLAMES configuration program

The program is started with the command configure typed at the UNIX shell prompt. This will bring up three windows. The control window contains a menu bar, status display, and message region. The largest window – the ‘mimic’ window – will be used to display a graphical representation of the FLAMES configuration being generated. A ‘basic sequence’ window provides a guide through a basic sequence of operations needed to generate a configuration for FLAMES.

## Opening files

The configuration program can read data from three types of file:

- Text files containing a description of the field in the standard format described earlier in this document. These are lists of targets and contain no fibre allocations. The default extension for these files is .fld.

- Configuration files in the standard FLAMES format used at ESO. These are PAF files used as input by the FLAMES observation software, and will invariably be created by the configuration program itself.

- Configuration files in the 2dF SDS format used by AAO. This format can be output by the FLAMES configuration program, and can also be used as input by it. However, these are not intended for use with FLAMES.

Note that an astronomer will always start by generating a text file describing the targets in a field. From this, a number of different configuration files can be generated in PAF (or SDS) format. If the astronomer or the operations staff want to check or modify a configuration then these files can be read back into the configuration program and new versions can be created.

To open a configuration text file select Open… from the File menu and select your file using the file selection dialogue that appears. By default, a file extension of .fld is expected for configuration text files. Alternatively, select the Open File option from the basic sequence window. If an input file contains a large number of targets, you may want to set a magnitude filter first to restrict the targets considered by the program to those within the magnitude range specified. To do so, select the Magnitude Filter option from the basic sequence window.

To open a PAF configuration file select Open PAF… from the File menu and select your file using the file selection dialogue that appears.

To open an SDS configuration file select Open SDS… from the File menu and select your file using the file selection dialogue that appears. By default, a file extension of .sds is expected for SDS configuration text files.

On opening your file the status display will show a summary of information about the field, and the objects in the field will be drawn in the mimic window. In the case of a text file no fibre allocations will be present, so the fibres will all appear in the mimic display at their park positions. A PAF or SDS file may already include fibre allocations and these will be shown on the display. To remove any existing allocations in order to start from scratch use Remove Allocations from the Commands menu.

## Converting Files

The configuration program can be used as a way of converting configuration files between text and SDS or PAF formats, in either direction.

To convert a text configuration file to a PAF configuration file, use Open… from the File menu to open the file, then use Save or Save As… to save the file in PAF format. This sequence can be performed non-interactively using the –d switch on the command line when configure is invoked. (See the section on running in batch mode.)

To convert a PAF configuration file to a text file, use Open PAF… from the File menu to open the file, then use List… to output the file in text format. The output file produced by List… is a valid configuration text file containing the unallocated objects from the configuration file. (List… always uses an equinox value of J2000 when writing its output files – for FLAMES this is the only allowed value.) It may also include a listing of the fibre allocations in the form of comments in the listing.

## Setting the Wavelength

Before allocating fibres it is important to set the desired observing wavelength. It is possible for an allocation that is valid at one wavelength to be invalid at another wavelength. Therefore, use the Set Wavelength… option in the Commands menu to set the desired central wavelength of observation.

## Setting the Field Plate

For some instruments, you may want to configure for a particular plate, for example when running online during a telescope run, or checking a field for the telescope. This is necessary because both plates always have different fibre and astrometry information. For FLAMES, it has been decided that any configuration must be valid for either plate. The plate data used by FPOSS is therefore a combination of the data for the two actual plates (so a fibre broken on either plate will show up as a broken fibre, for example).

To set the desired plate use the Set Field Plate… option in the Options menu. The default is to use plate 0, but this can be changed using the –p command line option. FLAMES can only accept plate 0.

## Overview of the Allocation Procedure

Automatically allocate fibres by selecting Allocate… from the Commands menu. This will bring up a dialogue box with a number of parameters controlling the allocation process. The default parameters will normally be suitable. During the allocation process, which will typically take under a minute to complete, a progress window will report on the progress of the allocation, and the mimic window and status display will be updated as new fibres are allocated.

The default is to leave no fibres unallocated if possible, but you can specify a number of fibres to be left unallocated for sky targets. Once the object allocation is complete you can assign these to sky positions. These can either be generated on a standard grid (using Allocate Sky Grid… from the Commands menu) or they can be supplied in the input file. Alternatively, you can add sky positions manually.

Once the allocation is complete, it will be checked for validity at the current position. As an extra step, you should select Check Over HA Range from the commands menu to check the validity of the field over a range of hour angles (the default is to check for plus or minus four hours from the meridian on the date set by the UTDATE field in the input file).

Saving the configuration as a PAF file will now give you an input file for the FLAMES positioner. You should ensure that all the available FACB guide fibres were selected.

## The Allocation Procedure in detail

The above recipe will suffice for the majority of fields. The hardware constrains the fibres to an angular limit from the radial direction which is instrument-dependent, but fibres are allowed to cross multiple times and in most cases all of them can be allocated to targets.

There are two basic allocation algorithms available, and these can be selected using the allocation dialogue panel – this can be seen in Figure 19. These are the ‘Oxford’ algorithm, also known as the ‘iterative’ or ‘cones’ algorithm, and the ‘Taylor’ algorithm, which has a number of variations such as ‘straightest target’, ‘hardest target’, ‘most crowded target’. There are then two additional options that you might want to consider, the tentative deallocation option and the fibre uncrossing option. You can also specify how many sky targets you want allocated to the various fibre types.

In general, the main determinant of which algorithm to use is the number of targets you are trying to deal with. If you have more or less the same number of targets as you have fibres, then you will generally get the best results using the ‘Oxford’ iterative algorithm, bypassing tentative deallocation, and with a final uncrossing pass. If you have a large number of targets, you may find this too slow an algorithm. In this case, you are usually best off with the ‘hardest target’ variation of the ‘Taylor’ algorithm, followed by both a tentative deallocation pass and an uncrossing pass.

Experienced users will have their own preferred recipes. Those prepared to spend time in the search of the ultimate configuration may use the individual allocation and deallocation facilities offered by the user interface, allocating and deallocating fibres by hand.

### The ‘Oxford’ algorithm

The algorithm currently used for 2dF (developed by Gavin Dalton at the University of Oxford) is highly optimised and has proven to give the best results for ‘typical’ fields, particularly when the number of targets is roughly the same as the number of fibres. Since this is not generally the case for FLAMES, this algorithm is not the normal default algorithm used for FLAMES configurations. This algorithm is invoked by selecting the iterative target allocation option from the allocation dialogue. After an initial allocation it searches down a tree of multiple fibre swaps, looking for swaps that give increased allocations. The algorithm is not unlike a chess program. The tree search terminates at a depth of ten swaps where it has been found, empirically, that the expenditure of CPU time required to deepen the search is not rewarded by increased allocations.

It is worth describing the operation of the algorithm in more detail. It is sometimes described as the ‘cones’ algorithm, because the concept of a cone with a target at its apex is an important part of the algorithm. Imagine a target somewhere in the field. Simply because of the limitations on the amount a fibre is allowed to bend as it leaves its pivot point, there are only a limited number of fibres that are physically able to reach this target. Imagine drawing the two extreme fibres out of all these, connected to the target in question. Combined with the sector of the pivot circle between the pivot positions of these two cones, these two extreme fibres delineate a ‘cone’ with the target at its apex. Now, the important point is that this cone will usually include other targets, and the fibres that originate within the cone are able to reach other targets outside the cone. However, the only fibres that can reach the targets in the cone are those originating within the cone. Imagine that there are just as many fibres originating in the cone as there are targets in it. (This will be the case where there are roughly as many fibres as targets, all evenly distributed.) This means that if there is no excess of fibres over targets for this cone, then it is going to be important that none of the fibres are allocated to targets outside the cone. If there is an excess of targets over fibres for the cone, it becomes particularly important to allocate all the cone fibres to targets within the cone.

The algorithm therefore considers all the possible cones formed by targets within the field, and sorts them into an order of importance, regarding the most important as those with the biggest excess of targets over fibres. The targets defining these cones are the ones it attempts to allocate first. The initial allocation pass is quite simple. The algorithm takes all the highest priority targets (those of priority 9) and sorts them into order of importance based on the target/fibre ratio for the cones they define. It then attempts to allocate targets to fibres in order of importance. It then repeats the process, this time including targets of the second priority rank (ie it now handles priorities 8 and 9). And so on, each time including lower priority targets, until it has done a pass with all targets.

This allocation process is quite fast, but generally leads to a somewhat unsatisfactory configuration, with a number of allocated fibres blocking the allocation of other currently unallocated targets. The algorithm then enters a ‘swap’ phase. In this phase, it attempts to improve the allocation by tentatively deallocating fibres. It takes an unallocated target (again, working in the ‘cone’ order of importance). Clearly, with the configuration as it stands, there is no fibre that can be allocated to this target. So it looks at each fibre that can be allocated to the target – at least on physical bend angle grounds – but is currently allocated. If there is one that can be deallocated from its current target and immediately allocated to the unallocated target, then it does so. It now has a different unallocated target. It may be that this slightly modified configuration will allow a fibre that was parked to be allocated immediately. If so, this is an immediate gain. If not, the algorithm just starts again with the new unallocated target, looking to see if there are currently allocated targets that can be swapped with it that might lead to a gain of a parked fibre. There is a safeguard that prevents the code from making ‘false’ gains – gains obtained by allocating more than one low priority target at the expense of a high priority target. This ‘swap’ phase is potentially quite time consuming,  particularly if there are a large number of unallocated targets – so time consuming that you may want to avoid the iterative algorithm if you have a large number of targets. You can cancel it at any time, should it be taking too long, so there is no reason not to try it to see whether you regard the time needed as unacceptable.

### The ‘Taylor’ algorithm

For fields with a large number of target objects – far more than the number of fibres – the swapping phase of the iterative target algorithm can take too long for this algorithm to be used successfully. For such fields, better results are obtained by an algorithm due to Keith Taylor. This operates in a number of different modes, and is invoked by selecting ‘by most crowded’, ‘by hardest target’ or ‘by straightest target’ in the allocation dialogue. The default algorithm for FLAMES is the Taylor algorithm in its ‘hardest target’ mode.

This algorithm is relatively easy to understand. It assumes that the hardest pivots to allocate will be those that can be allocated (on grounds of physical constraints, such as fibre bend angle, fibre length, etc. and target/fibre compatibility) to the fewest targets. It therefore allocates the pivots to targets in order of decreasing difficulty, starting with the hardest pivot to allocate. Having decided which pivot to allocate, it then has to pick a target for it. It will allocate to the highest priority target available for that pivot. If more than one target of the same priority can be allocated, then the decision depends on just how the algorithm was invoked.

If ‘by hardest target’ was selected, it picks the target that can be assigned to the fewest fibres. These targets are going to be hard to allocate, so it tries to allocate them as soon as possible.

If you selected ‘by most crowded’, it picks the possible target with the least average distance to other targets. This is suitable for fields that have areas that are particularly crowded, and in particular, for cases where the user is trying to get a number of configurations that will, together, cover a large target list. (The ‘hardest target’ option will often allocate a greater number of targets for a single configuration, but will do so by ignoring crowded targets, leaving these to be covered by the final configurations in a set, which is unsatisfactory.)

If you selected ‘by straightest target’, it picks the target that will bend the fibre the least. This option produces allocations with the minimal number of crossed fibres, which can be a consideration if exposure times are small and it is important to be able to reconfigure the field as quickly as possible.

You should be aware of the following quirk of the Taylor algorithm, in all its different modes. It looks first for the hardest fibres to allocate. If there are very few targets, then in general the hardest fibres to allocate will not be the fibres closest to the targets. This means that in these circumstances, the algorithm will tend to allocate fibres at some angular distance from the targets, leading to a rather ugly ‘skewed’ allocation. If you have very few targets, you are probably better off using the ‘Oxford’, iterative, algorithm.

### Tentative deallocation

If you selected tentative deallocation in the allocation dialogue, the program performs a pass where it tentatively deallocates each allocated fibre in turn and then sees if this allows more than one other allocation to be made. If so, there has been a gain from this deallocation. The option takes a little time, but can often squeeze a few extra allocations out of a field. This option is normally only used when the main allocation is done using the Taylor algorithm, since the swap phase of the Oxford algorithm is really a much more comprehensive case of the tentative deallocation pass.

### Fibre uncrossing

The optional final ‘uncrossing’ pass looks at all pairs of allocated fibres which cross to see if they can be reversed. This is important, as reducing the number of fibre crossings in the final configuration produces a significant reduction in the field setup time by reducing the number of fibres that must be parked in transit. Note that this reduction is obtained without constraining the allocation itself.

Uncrossing is not supported at the moment if the Taylor algorithm is being used. However, it is possible to make an allocation using the Taylor algorithm and then select uncross fibres… from the commands menu. This invokes a modified a relatively fast form of the Oxford algorithm that only performs the uncrossing pass.

### Sky allocations

The allocation dialogue allows you to specify how many sky targets are to be allocated to each fibre type. (This is not useful in 6dF, where sky targets can only be allocated to the FLAIR spectrograph fibres, but it can be useful for 2dF, ensuring that each spectrograph gets a roughly equal number of sky fibres.) This is implemented rather simply. At the end of the allocation, the program looks to see if there were explicit sky targets specified in the target list (either in the original input file, or through the generation of a sky grid or the explicit nomination of a given position as a sky position).

If there are sky targets available, the program attempts to allocate the specified number of fibres of each type to the available sky targets. If necessary, it will deallocate low priority targets in order to free up enough fibres for the specified number of sky targets.

If no sky targets are available, the program assumes that you plan to generate a sky grid or specify sky targets explicitly, and it simply deallocates fibres allocated to sufficient low priority targets to ensure that the specified number of fibres are unallocated.

## Target Priorities

The targets are allocated in order of priority, with a numerical value of 9 being the highest priority and 1 being the lowest. To ensure all guide fibres are allocated it is best to give guide targets the very highest priority values.

## The allocation window

When Allocate… is selected from the Commands menu, the allocation window appears. This allows you to control the details of the allocation procedure. The window is shown in Figure 19.

The parameters here control the various steps of the allocation. The default is to allocate the maximum number of targets and optionally leave a number of fibres for subsequent sky allocation.

- Target selection allows the selection of the allocation algorithm as described in the previous section on the allocation algorithm in detail.

- Fibre uncrossing controls aspects of the fibre uncrossing scheme. The default is to uncross at the end of the allocation, which will suffice for most fields. If there are many priority levels of object, one can sometimes gain in allocations by selecting uncrossing after each priority pass, though this is more time consuming. It is also possible to uncross existing configurations either here or by selecting Uncross Fibres in the Commands menu and then proceed to allocate extra objects without removing the existing allocations.

## Sky Fibres

It is important to have sufficient sky fibres allocated.

You can supply sky positions in the input catalog – for example, positions which have been carefully checked on images to ensure absence of bright objects. To assign to these, make sure that you have suitable values set in the boxes in the allocation dialogue that allow you to specify the number of fibres of each type that are to be allocated to sky targets.

If you do not have sky positions in your input catalog, you should still use the same boxes in the allocation dialogue that allow you to specify the number of fibres of each type that are to be allocated to sky targets. If there are no sky targets at all in the target list, the specified number of fibres of each type will be left unallocated in the normal allocation process. You may then create a grid of sky positions and allocate these positions automatically by selecting Allocate Sky Grid… from the Commands menu.

Finally, arbitrary sky positions can be assigned interactively (see below).

It may be desirable to check the positioning of sky fibres that have been automatically allocated or added by hand, to ensure that these are not contaminated by stray objects, particularly brighter stars. This can be done by selecting List… from the Commands menu, and checking the Allocated Sky as DSS input button. This will list all allocated sky positions as J2000 coordinates to a file (the default file name is the same as that of the input file, but with the extension .dss) which is in the correct format to be read by the commonly available StScI getimage program. The sky positions are named Snnn in the .dss file, where nnn is the fibre number. The content of the sky fibres can then be conveniently checked using a FITS aware visual browser (eg the visual schnauzer in xv).

## Manual Allocation of Fibres

It is also possible to manually allocate individual fibres to objects. If you are using a priority scheme for allocating targets then you can highlight all allocated or unallocated targets within a given range of priorities using the Highlight Targets… option in the View menu.

Manual allocation is performed by interacting with the mimic display as follows:

- Select a fibre by clicking on its button – it will be highlighted in red.

- Select an object by clicking on it – it will also be highlighted in red.

- Select Allocate Fibre from the Commands menu.

The validity of the resulting configuration will be checked, and a progress dialogue will be displayed as this happens. If the resulting configuration is valid the allocation will be made and the display updated. If it is not valid, an error dialogue will appear.

There is a short cut in the manual allocation procedure that avoids the use of the menu as follows:

- Select a fibre by clicking on its button.

- Move the cursor over the desired object and click the middle mouse button.

It is possible to manually allocate fibres and then selct ‘uncross first then allocate extras’ in the allocation dialogue window to automatically assign the rest of the fibres while preserving the manual allocation.

It is also possible to deallocate fibres manually as follows:

- Select a fibre by clicking on its button. You need to position the cursor carefully to ensure you select the fibre rather than the object. Make sure the fibre button is highlighted in red. This operation is easier if the display is zoomed.

- Select Deallocate Fibre from the Commands menu.

Sky positions, either included in the input configuration file or generated as a sky grid, are allocated in the same way as objects. However, there is an additional facility that allows you to create an arbitrary sky position anywhere in the field. To do this, select a fibre in the normal way with the left mouse button. To assign it to a sky position use CTRL + middle mouse button. This creates a sky target in the configuration under the cursor and attempts to assign a fibre to it and is most useful for assigning miscellaneous unallocated fibres to nominal sky positions.

## The Mimic Display

The display can be zoomed to magnifications of 2,4, or 8 times its normal scale using the Zoom menu. The zoomed display can be scrolled using the scroll bars.

It is also possible to zoom the display by a factor of two about any selected point by clicking the right mouse button with the cursor positioned at the desired centre.

In the display, reference stars are shown as large red circles, target objects as small black circles and sky targets as blue squares. VLT guide stars are shown as large cyan circles. Fibre buttons are shown in different colours to distinguish between different fibre types. Disabled or non-existent fibres are shown in grey. Selected objects and fibre buttons are shown in blue. Information about individual objects or fibres in the display can be examined by double clicking on the objects or fibre buttons.

To locate an arbitrary fibre button enter the pivot number in the fibre information popup (which comes up when any fibre is double clicked on) and press return – this selects the requested fibre.

The display can be printed by selecting the Print… command from the File menu. A postscript version of the mimic window will be generated which can either be sent directly to a printer or saved as a file, according to selections in the resulting dialogue box.

## Saving a Configuration

After completing the allocation process and checking its validity, the resulting configuration can be saved as a PAF file by using Save or Save as PAF… from the File menu. The resulting PAF file is in a form suitable for use by the FLAMES observing system.

It is also possible to save the configuration in the AAO’s SDS format, using the Save as SDS… option in the File menu.

A text file listing the fibre allocations and/or the unallocated objects can be obtained by selecting the List… option from the File menu.

## Batch Mode

On most modern workstations a configuration takes less than a minute. It is recommended that the configure program be run interactively. However, there is a limited feature for using it in batch mode. To make configure read in the input configuration file filename.fld and allocate it automatically, use the command:

configure –a –f filename.fld

(Note: the command to start configure may be different on your system.) This will produce two new files, filename.sds and filename.. The first can be fed back into the program in interactive mode to show how the fibres have been allocated. The second is a text file that contains details of the pivot allocation and can be processed by some other program to analyse the way the field has been configured) Because FLAMES configurations need additional information, such as the guide star and fibre combination to use, which cannot be specified on the command line, batch mode is probably not useful for FLAMES.

## Command Line Switches

The configuration program supports a number of command line options:

-a	Invokes batch mode.

-f filename	Specifies the initial input file.

-I instrument	Specifies the instrument to be used. This is only useful if the program has been configured with support for a number of different instruments, in which case it eliminates the initial dialogue where configure asks you which instrument to use. For FLAMES one would use –I FLAMES.

-k	Disables fibres which go to a particular spectrograph. (this is being revised for FLAMES and doesn’t work at present.)

-p N	Sets the initial plate. (This is not usually relevant for FLAMES.)

There are also a number of options that can be used to perform specific actions when invoked with the –f option:

-i	Specifies an ASCII file to read with a set of fibre number, fibre name pairs. This file will be used to perform a specific set of allocations with no checks whatsoever. It will then write an output  and exit.

-d	Instructs configure to read in the .fld file, write out a .sds file with no allocations and then exit. This is useful if there are targets in the file that you wish to inspect with 2dF’s focal plane imager. (This may not be relevant for FLAMES.)

-s	Instructs configure to interpret the argument of the –f flag as a .sds file. (For FLAMES this will be changed to a PAF file.) This file is read in an  auto-reallocation pass is performed, followed by an additional allocation pass  and an allocation of remaining fibres to the grid of sky positions. This option was introduced to 2dF for the Galaxy Redshift Survey, but may also be used for FLAMES to reprocess a configuration just before observing.

## Reference

You will find detailed descriptions of most terms in the following user manual. It incorporates a glossary and a discussion on Observation Blocks.

P2PP User’s Manual  - David Silva, ESO Support Group VLT-MAN-ESO-19200-1644/2.1B

# Obtaining and installing the program

FPOSS can be obtained, as are other ESO observing tools, via the web, and can be found under the address:

http://www.eso.org/observing/p2pp/OSS/FPOSS

The FPOSS program is distributed in the form of a binary file suitable for Solaris 2.6 (SunOS 5.6), HP-UX or Linux (built under Red Hat 7.2) as a gzip’ed tar file Create an empty directory and gunzip it, then un-tar it into the same directory. The directory will now contain the program itself, configure, together with a number of .tcl files it uses and a couple of additional files that provide details of the FLAMES configuration and bitmaps used by the program. It will also contain two sub-directories that contain the Tcl/Tk files needed to run FPOSS. (The final version of the program will be obtainable from the ESO Web site. Users should make sure they are running the latest version of the program. In particular, they should make sure they are using the latest version of the instrument status file fpInsStatus.sds, since this has the most up to date configuration for the fibres – including details of any broken fibres.)

The program is a straightforward tcl/tk program, which is why it is relatively easy to port to various different platforms. The main requirement is that tcl/tk should be available for the target platform.

# The Menus in detail

This section describes the menu contents for the all the menus available in the main control panel – the File, View, Options, Commands, and Zoom menus. All the features of the program are available through these menus.

## The File menu

Figure 1 shows the File menu. Note that this is shown when the configuration program is being used to configure FLAMES. For other instruments, SDS file format is the default rather than PAF, and this is reflected in the menu options.

Figure 1 The file menu

Open	Puts up a file selection dialogue to read in a new list of targets from a text input file (usually a .fld file). If a file is already open and has not yet been saved, you will be asked if you want to save it before opening the new file.

Open PAF	Puts up a file selection dialogue to open a PAF format file (usually a file saved as the result of a previous run of FPOSS). This will put the program back to the state it was in when that file was saved, at least so far as fibre allocations and VLT guide probe settings are concerned. PAF files are not normally saved with details of unallocated targets included, so this information will not normally be restored. If a file is already open and has not yet been saved, you will be asked if you want to save it before opening the new file.

Save as PAF File	Puts up a file selection dialogue allowing you to save the details of the current configuration in PAF format.

List…	Puts up a dialogue box allowing you to specify what details should be included in the list file to be created, and then puts up a file selection dialogue box to allow you to specify the name of the list file. This can be used to generate catalogues that can be read by Skycat.

Print…	Creates a printed version of the mimic display window.

Exit	Exits the program.

## The View menu

Figure 2 shows the View menu.

Figure 2 The view menu

Select Targets	Puts up a dialogue that allows you to highlight target objects of different types. For example, you can choose to highlight tagets in a given priority range, or targets that are classified as UVES targets. Highlighting is quite subtle and if there are a large number of targets even a highlit target may not show up sufficiently. So this dialogue also allows you to redraw the mimic display with all targets magnified. It also has an option that indicates all highlit targets with large blue arrows – the result is unsubtle, but quite obvious.

Use grey shades for display	Switches from using colours to differentiate between fibre types to using different shades of grey. Some users find it easier to distinguish shades of grey rather than colours. This option is only active if grey shades are not already being used.

Use colours for display	Switches from using different shades of grey to differentiate between fibre types and using colours. Most users find it easier to distinguish colours rather than shades of grey. This option is only active if colourss are not already being used.

Show Argus settings	This puts up a dialogue allowing you to specify the ARGUS position angle, in degrees, and to choose between the two different ARGUS scale factors (1:1 or 1:1.67). If you change the position angle, this will invalidate any current allocations and will cause recalculation of all the target positions.

## The Options Menu

Figure 3 shows the Options menu.

Figure 3 Options menu

The options menu provides control over a number of possible options for the program.

Set magnitude filter…	Puts up a dialogue box that allows you to change the magnitude range used to filter the input target list.

Change Field Label	Allows the label for the field to be changed.

Select Fibre Combination	Puts up a menu showing all the fibre combinations supported by the instrument, allowing one such combination to be specified for use when fibres are allocated. This allows the same input configuration file to be used to generate a number of different configurations depending on which instruments are in use.

## The Commands Menu

Figure 4 shows the Commands menu.

Figure 4 The commands menu

The Commands menu allows you to invoke most of the operations that FPOSS can perform.

Allocate	Performs a complete fibre allocation. It puts up the allocation dialogue that allows you to modify the parameters used by the allocation algorithm, and then starts the allocation process.

Auto-reallocate	Invokes the allocation algorithm in a mode where it modifies the current allocation to allow for broken fibres. This is useful when an existing allocation has been read in from an SDS file but the instrument setup has changed since the allocation was made.

Check Allocation	Performs a validity check on the current allocation. This is useful when an existing allocation may have become invalid – for example, because the observation time has been modified.

Check Over HA range	Puts up a dialogue allowing you to check the current allocation over a range of dates and hour angles.

Allocate Sky Grid	Automatically allocates a grid of sky targets. This is an alternative to providing a specific set of sky targets in the input target file, but of course the program has no way of knowing what parts of the target field are really clear and suitable for use as sky.

Uncross Fibres	Invokes the allocation algorithm in a mode that merely attempts to exchange fibre/target allocations in such a way as to mimimise the number of fibre crossings.

Set Hour Angle	Puts up a dialogue box allowing you to specify the hour angle for the observation. After a change such as this you should check the validity of the allocation (if there is one) using Check Allocation.

Remove Allocations	Removes all the current fibre/target allocations, if any. A dialogue box is put up asking you to confirm that you want this doing.

Allocate fibre	This is only active if a fibre has been selected (by clicking on in the mimic display) and a target has been selected (also by clicking on in the mimic display). It causes that fibre/target allocation to be made and verified. This can be useful as a check on why the algorithm did not make what appears to be an obvious allocation – select the fibre and target in question and use this option to allocate them explicitly. You will often get an error message explaining why the allocation cannot be made.

Deallocate fibre	Only active if a fibre has been selected (by clicking on it in the mimic display). If the selected fibre has been allocated to a target, this causes it to be deallocated and returned to its park position.

Show fibre info	Only active if a fibre has been selected (by clicking on it in the mimic display). Puts up a window giving details about the fibre.

Set field center	Moves the field center so that the field is now centered at the place last clicked on the mimic display. So the idea is that you click on the mimic display at the required field center, then select this menu option. You will see a dialogue box asking you to confirm this change. This change will invalidate any current configuration.

Deallocate Broken Fibres	Removes any allocations that involve broken fibres. You can only get such an allocation by reading in a configuration (from an SDS or PAF file) that was made with a different instrumental setup file.

Deallocate Sky Fibres 	Removes any allocations that involve sky targets. (The name of the menu item is a little misleading – it is the target that is sky, rather than the fibre, although it is true, for example, that all ARGUS fibres are sky fibres.)

## The Zoom menu

Figure 5 shows the Zoom menu.

Figure 5 The zoom menu

This menu permits you to select a display according to a range of scale factors. You can reduce the image size, as well as expand it from the usual size. The zoom menu just presents a set of 6 zoom values that are generally useful. More extreme values can be obtained by right-clicking in the mimic display. (Note that there is no dialogue that allows a completely arbitrary zoom factor to be specified.)

# Appendix A: A tour of  FPOSS

You should try this out using the test configuration file supplied with the program. A number of test files are supplied with the configuration. For your first tour through the program, you should use the supplied file N3201-00.fld. This is a fairly typical input file. This section may seem quite long, but that is mainly because it has a lot of screen shots in it. Actually running through the tour should only take a few minutes.

## Starting up

The configuration program is invoked in the same way as any UNIX program, by typing its path name and appending any necessary parameters. If the configuration program is in the user’s default directory, all that is required is the command:

./configure &

(The ./ prefix is needed because security-conscious users will not have the default directory in their path, and the final & is because it is usually most convenient to run the program in the background.)

The program will start up, displaying three windows. There will be a main control panel (Figure 7) and a window that will eventually show a ‘mimic’ display of the field plate with targets and fibres. Initially, this will just show a number of concentric circles of varying radii (Figure 8). There will also be a ‘basic sequence’ window that is intended to guide you through the program (Figure 6).

Let’s take a quick look at each of these windows, although at the moment they aren’t very interesting.

Figure 6 The Basic Sequence Window

For most uses of FPOSS, you will be able to control the configuration process almost entirely from the basic sequence window. The idea is that you press each of the buttons down the left hand side of the window in sequence. That is what we will do first in this tour of the program. Later we will look at some of the other options, using the menus in the main control panel..

Figure 7  The control panel

The control panel contains a number of menus, and a large number of fields where information can be displayed, together with a scrolling region at the bottom for messages. Note that the fields are not editable, even though they might look like it.

Figure 8 The initial mimic display

The mimic display at present only shows a number of concentric circles. If you move the cursor over these,  the left part of the status bar at the top of the window will tell you what they are. The black circle at the center is ARGUS. (Actually, ARGUS is rectangular, but the field plates have a circular hole in the middle to allow for ARGUS, and this hole (where fibres cannot be placed, of course) is circular. The yellow circle marks the 20 arcminute diameter circle, the blue circle marks the furthest out that a fibre can be placed. The black circle marks the pivot circle – the point where the fibres emerge from their retractors around the field plate. The red circle marks the 29 arcminute diameter – VLT guide stars should not be chosen outside this red circle.

## Setting a magnitude filter

For files that have a large number of targets you will probably want to set a magnitude filter to reduce the number of targets to a manageable number. This is certainly true of the N3201-00.fld test file, which contains thousands of targets. (So many that if you try to open it without having set a magnitude filter the program will warn you about it.)

So, this is where we start out through the basic sequence window. Click on the ‘magnitude filter’ button. You will see a dialogue prompting you to set a magnitude filter range.

Figure 9 Magnitude filter dialogue box

For the  N3201-00.fld test file, selecting a magnitude range between 16 and 17 will give a suitable sample of targets for our purposes. Fill in the two entry boxes and click on the ‘Apply’ button. Once you do that, the dialogue box changes to indicate the new settings.

Figure 10 Magnitude selection made

You can now click the ‘close’ button and the dialogue box will go away. You will find that the basic sequence dialogue window has changed to reflect what you’ve just done.

Figure 11 Magnitude filter selected

The line to the right of the magnitude filter button now indicates the magnitude filtering you selected, and there is a small red tick to the immediate right of the button. As you work through the basic sequence, each item should get ticked to remind you of how far you have progressed.

## Opening the data file

Now click on the ‘Open File’ button in the basic sequence window. This will bring up a file selection dialogue box. This will look something like this, depending on what files you have in your directory:

Figure 12 Open file selection

Select N3201-00.fld and click on the Open button. Now things start to happen. The target list in this file will be read in, filtered for targets between 16th  and 17th magnitude, and the program now has something to display on the mimic window. You will also notice that there is information at last in the various fields of the main control panel.

Figure 13 Control panel with file open

As shown above in Figure 13, the details of the field are now displayed in the main panel, which will now show the allocation statistics for all the various supported categories of fibre. Note that ‘TEST’ fibres – which are not normally allocated by FPOSS – can in principle to allocated to any target type other than guide fibres, which is why the program shows the total number of targets that can be allocated to TEST fibres as the sum of all the targets other than guide fibres. Some classes of target, such as sky targets, can be allocated to more than one fibre type and so will be counted more than once in the ‘available targets’ section of the display.

Now that a configuration file has been read in, the targets are displayed in the mimic window, and the fibre positions are drawn, as shown in Figure 14. Since no fibres are allocated as yet, all the fibres are shown at their park positions, just outside the 25 arc-minute radius, shown as a blue circle on the mimic display. The other two circles drawn in the mimic display show the 25 arc-minute radius circle (in yellow) and the 29 arc-minute radius (in red). These circles are the innermost and outermost, respectively. A compass at the top-right of the display shows the East and North directions, and the instrument is displayed in the top left corner. The black circle at the centre of the display shows the position of ARGUS. The orientation of the display is the default orientation, with North pointing to top-right and East pointing to top-left. If the input file contains a line specifying an ARGUS position angle (which the N3201-00.fld file does not) then the field orientation may differ.

There are a number of different types of fibres used by FLAMES, and each type is shown in a different colour, althought these are not always easy to distinguish If you place the cursor on top of a fibre the fibre type is displayed in the top information bar of the mimic panel.

Note that other features of the mimic display, such as the meaning of the circles, are identified in the same way by moving the cursor upon them. The information appears in the top information bar.

Figure 14 Mimic display with new field

## Selecting the VLT guide star

The next item in the basic sequence window is ‘Select guide star’. If you are using the VLT then you need a VLT guide star that can be used for both the VLT guiding and adaptive optic systems. You must have such a guide star selected, or FPOSS will not allow you to save your configuration. The input file identifies such guide star targets as type ‘G’ and they are shown in dark blue on the mimic display, drawn rather more solidly than the other targets. The test field has one right at the edge of the 28 arcminute circle, at about 2 o’clock. We will use this one.

(If you have difficulty distinguishing the different target types, there are a few features built into the program to help – some users may find a grey-scale display easier to work with, and there is a menu that can be used to highlight targets of various types. This is described in more detail later. For the moment, this particular guide star should be easy enough to find.)

If you click on the ‘select guide star’ button in the basic sequence window, all you get is a message telling you to double click on a VLT guide star. The program can’t do this for you, or even put up a dialogue to help. You have to double click on the guide star you want to use. If you double click on any target star, FPOSS puts up a window giving details about the target. If the target is a VLT guide star, FPOSS also puts up two circles that represent the two possible positions for the VLT guide probe pivot if it is to be assigned to that particular target star. If you double click on that guide star at 2 o’clock, the mimic display will show these, as in Figure 16.

The target information window that also pops up is shown in Figure 15.You don’t really need this at the moment, so you might as well just click the ‘cancel’ button which will make it go away. If you don’t see the two pivot positions appear in the mimic display, you should check that the target type shown really is ‘VLT Guide star’. If it isn’t, you got the wrong target.

Figure 15 Target information window

Figure 16 Guide star pivot positions

Click on one of the blue circles. You will see the shadow of the VLT guide probe drawn in the display. You can click on the other circle to change to the other position for the probe. Once you click on one of the pivot circles, that selects the probe position and that is all you need to do. If you want to change to another guide star, double click on it and repeat the process.

For the moment, let’s assume you choose the upper pivot position and click on it. The mimic display will show this as in Figure 17. Because the guide probe needs to intercept the light from its target star it has to be above the field plate and so no light will reach fibres below the guide probe. This means that there is a large guide probe ‘shadow’ on the plate, and target objects in this shadow region cannot be observed.

You can see that there are advantages to having the guide probe target as close as possible to the edge of the field. Targets under the guide probe shadow cannot be used and will not be allocated to fibres by the program. If you click again on the same pivot position, the guide probe shadow changes to an outline – this allows targets under the shadow to be selected using the mouse, something that is mainly useful if there is an alternative guide probe target in the shadowed area.

Figure 17 Guide probe position selected

## Selecting the Fibre combination

There are only a limited number of valid fibre combinations that can be used with FLAMES, and you need to select one of them. Click on the ‘Fibre combination’ button in the basic sequence window, and the following dialogue will appear:

Figure 18  Fibre combination dialogue

You need to select one of the allowed combinations. For the moment, select ‘Medusa + UVES’. Click on the radio button (the diamond) to the left of the line that says ‘Medusa + UVES’ and click on the ‘Apply’ button.

If you select a fibre combination that includes ARGUS sky fibres, you will be presented with a dialogue box that asks you to specify the ARGUS scale factor and position angle.

The mimic display will change once you hit the ‘apply’ button. All the fibres except for those of the type selected are removed from the display, and the fibre colours change. Now that there are fewer fibre types in use, a different set of colours are used. The main fibre type (Medusa)  is now drawn in red, and the secondary type (UVES) is drawn in blue. For all the

Combinations, guide fibres (the FACBs) are automatically included, and these are shown in green.

Now, we are in a position to allocate fibres to targets

## Performing the Allocation

Click on the ‘Allocate’ button in the basic sequence window. You will get a dialogue box that presents you with a number of options that can be used to control the details of the allocation algorithm.

Figure 19 Allocation dialogue

If you accept the default options and click the OK button, the allocation process will begin. A progress box will be displayed giving some indication of what is going on, and eventually the allocation will be complete. As fibres are allocated, they are drawn in their new positions on the mimic display, and the final result will look something like this:

Figure 20 Allocation completed

And that’s it. One field allocated. Now all you have to do is save it, and that’s the next step in the basic sequence.

## Saving the allocation

Click on ‘Save as PAF’ in the basic sequence window, and you get a standard file save dialogue. The file name is generated automatically according to FLAMES conventions, and you should not normally change it. However, you will need to note it down because this will have to be supplied to P2PP. The file name is based on the label for the field, the fibre combination in use, and the time of day. This ensures that a unique, informative, name is used for the file.

Click on the ‘save’ button, and the file will be written.

Figure 21 PAF file save dialogue

And that has completed a basic run through FPOSS. The basic sequence window should now look roughly as follows:

Figure 22 Basic sequence completed

You’ve got everything ticked. Well done. Now you can exit the program using the ‘Exit’ button, or you can go back to the start and perform a new configuration with a new input file.

## Things not in the basic sequence

There are a couple of things you may want to do as a matter of course when you run FPOSS, but which are not included in the basic sequence. One is to set the wavelength to be used, and the other is to check the range of hour angle for which the configuration is valid.

### Changing the central wavelength

The program needs to know a wavelength to use when calculating the positions of the targets on the field plate. If the wrong wavelength is used, then the dispersed images at significant hour angles will result in the fibres getting the wrong portion of the light from the object. To set the wavelength, we need to use one of the menus in the control panel.

Bring the control panel to the front, select the ‘Commands’ menu and select the ‘Set wavelength…’ item in the commands menu. A small dialogue appears:

Figure 23 Set wavelength dialogue

You can use this to set the configuration wavelength to the main wavelength of interest for your observations. Note that the guide star positions are calculated on the basis of a different wavelength based on the known characteristics of the FACBs and their filters. The ‘Set wavelength…’ dialogue does not affect this guide star wavelength. Note that the units used are Angstroms.

### Checking Allocation Validity

The allocation process results in a fibre configuration which is valid at a single time. However, small changes in the relative positions of objects as a result of refraction and other effects could make this configuration invalid at other times and different telescope positions. It is possible to change the Hour Angle using the Set HA… option in the Commands menu, and then check the validity of the allocation using Check Allocation, also in the Commands menu.

When FPOSS creates configurations, it does so by default using a set of tolerances for such things as button clearances that are more restrictive than those actually used by the positioner. These have been chosen so that in all cases tested, a configuration was valid (ie would not be rejected by the positioner) over more than a +/- 4 hour range in Hour angle. So unless you intend to observe at truly extreme hour angles, you do not normally need to make this check.

The Check over Range of HA… option in the Commands menu performs a check that a fibre configuration remains valid over a range of dates and telescope hour angles. The check should run through with Allocation OK reported in the message region for each position tested. Occasionally one or two fibres or buttons that are OK at the zenith will cause collisions at large hour angles. The simplest procedure is to manually reassign these as extra sky fibres, although it is usually possible to manually adjust the configuration to preserve the target allocation while removing the collisions. An option in the Check over Range of HA… dialogue can be used to get FPOSS to attempt this reallocation automatically.

Figure 24 HA validity check dialogue

Naturally, the configuration produced by the allocation sequence just worked through passes this default check. To illustrate a failure a configuration was generated using the actual positioner tolerances rather than the larger tolerances normally used by FPOSS. Now there is one rogue fibre that is positioned rather close to a button and at extreme hour angle values moves just close enough to be counted as a collision. If you run the validity test you get:

Figure 25 Hour angle test fails

By running the cursor over the fibres, you can easily pick up fibres 6 and 62, and you can see that the button on fibre 62 is indeed allocated rather closely to fibre 6. The easiest thing to do is click on fibre 62 then select the ‘Deallocate fibre’ command from the commands menu. If you run the test with the ‘attempt to reallocate invalid fibres’ button checked this has the same effect.

___oOo___